CISC SIMULATOR

Manual

V 1.0

Group 8

Md Shahjalal

Tianyou Bao

Xuzheng Lu

Contents

1 Introduction	1

1.1 Debugging Panel	1

1.1.1 Register Indicators Area	1

1.1.2 Memory Area	2

1.1.3 Controller Area	2

2.2 Classic Panel	3

2 Operation	3

2.1 Writing Values to Registers	3

2.2 Writing Values to Memory	3

2.2.1 Using Memory Address Register and Memory Buffer Register	3

2.2.2 Modifying the Memory Area	4

2.3 Executing Instructions	5

2.3.1 Executing Instructions Step-by-Step	5

2.3.1 Executing Instructions Automatically	5

3 Instructions Reference	6

3.1 Load/Store Instructions	6

3.1.1 LDR Instruction	6

3.1.2 STR Instruction	6

3.1.3 LDA Instruction	7

3.1.4 LDX Instruction	7

3.1.5 STX Instruction	7

3.2 Other Instructions	7

3.2.1 Halt Instruction	7

# 1 Introduction

This simulator is a simulation of a Complex Instruction Set Computer (CISC). Two panels are designed for the simulator.

## 1.1 Debugging Panel

Debugging Panel displays all the information about the Registers, Indicators, and Memory in the computer and can be written manually.

The panel is divided into three parts：

### 1.1.1 Register Indicators Area

The Register Indicators display the values of all kinds of registers.

- Click the 'W' button to manually modify the value of a register.

- Hexadecimal values are shown on the right.

| Type | Size(bits) | Number | Description |
| --- | --- | --- | --- |
| R0...R3 | 16 | 4 | General-Purpose Register |
| IX1...IX3 | 16 | 3 | Index Register |
| MAR | 16 | 1 | Memory Address Register |
| MBR | 16 | 1 | Memory Buffer Register |
| PC | 12 | 1 | Program Counter |
| IR | 16 | 1 | Instruction Register |
| CC | 4 | 1 | Condition Code |
| MFR | 4 | 1 | Machine Fault Register |

### 1.1.2 Memory Area

The Memory Area shows the address, the value, the Hexadecimal value, and the Assemble Code of each line on memory.

- The memory address pointed by the Program Counter will be highlighted.

- Double click to manually modify the binary value of a memory row.

### 1.1.3 Controller Area

The Controller Area integrates all function buttons and the instruction input box.

| Button | Description |
| --- | --- |
| Reload | Initialize the values |
| Save | Save inputs |
| IPL | Pre-load a program |
| Auto Run | Run instructions automatically |
| Single Run | Run instructions step by step |
| Pause | Pause the machine |
| Stop | Stop the machine |
| Restart | Restart the machine |

## 2.2 Classic Panel

The appearance and operational logic of the Classic Panel emulate the PDP-8 computer. Users will use switches to input and lights for indication.

The Classic Panel has not been finished yet and will be released in the next version.

# 2 Operation

## 2.1 Writing Values to Registers

Following the steps below to write a value to a register.

Step 1: Input a value into the box

Step 2: Click the 'W' button at right to write the value to the register

Step 3: Done! The value will be written to the Register.

Error handling:

- Input too long: Remove the excess bits from the left

- Input too short: Add zeros from the left

- Input is not binary: Pop up an Error window

## 2.2 Writing Values to Memory

Two methods are acceptable to write a value to the Memory.

### 2.2.1 Using Memory Address Register and Memory Buffer Register

Step 1: Input a value into the MAR box

Step 2: Click the 'W' button of MAR

Step 3: Input a value into the MBR box

Step 4: Click the 'W' button of MBR

Step 5: Done! The value of MAR will be written to the Memory, and the MAR will automatically change to the next address.

### 2.2.2 Modifying the Memory Area

Step 1: Double click the memory row that needs to modify

Step 2: An input window as the following will pop up. Input the value that needs to write to the memory

Step 3: Click the 'OK' button, and then the value will be written to the Memory.

## 2.3 Executing Instructions

Instruction can be executed step-by-step or automatically.

### 2.3.1 Executing Instructions Step-by-Step

Step 1: Store an instruction to the Memory

Step 2: Write the address of the instruction to the Program Counter (PC)

Step 3: Click the 'Single Run' button, and then the instruction will be executed.

- The Program Counter will automatically point to the next address of Memory.

- The Instruction Register will store the last executed instruction.

### 2.3.1 Executing Instructions Automatically

Step 1: Store instructions to the Memory

Step 2: Write the address of the starting instruction to the Program Counter (PC)

Step 3: Click the 'Auto Run' button, and then the instructions will be executed automatically.

- The Program Counter will automatically point to the next address of Memory after an instruction being executed.

- All the indicators will be continuously updated while the program is running.

Step 4: Click the 'Pause' button or the 'Stop' button to stop the program.

# 3 Instructions Reference

## 3.1 Load/Store Instructions

The basic instructions to load/store values from/to Registers or Memory. The binary instruction code format is as follows:

| Opcode | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Opcode: | 6 bits | Specifies the instruction |
| --- | --- | --- |
| R: | 2 bits | Specifies the General-Purpose Register |
| IX: | 2 bits | Specifies the Index Register |
| I: | 1 bit | Specifies Indirect Addressing If I =1, indirect addressing; otherwise, no indirect addressing. |
| Address: | 5 bits | Specifies the location |

### 3.1.1 LDR Instruction

| Instruction: | LDR r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 01 |
| Binary-Opcode: | 000001 |
| Function: | Loads Register from Memory |

### 3.1.2 STR Instruction

| Instruction: | STR r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 02 |
| Binary-Opcode: | 000010 |
| Function: | Stores Register to Memory |

### 3.1.3 LDA Instruction

| Instruction: | LDA r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 03 |
| Binary-Opcode: | 000011 |
| Function: | Loads Register with Address |

### 3.1.4 LDX Instruction

| Instruction: | LDX x, address[, I] |
| --- | --- |
| Octal-Opcode: | 41 |
| Binary-Opcode: | 100001 |
| Function: | Loads Index Register from Memory |

### 3.1.5 STX Instruction

| Instruction: | STX x, address[, I] |
| --- | --- |
| Octal-Opcode: | 42 |
| Binary-Opcode: | 100010 |
| Function: | Stores Index Register to Memory |

## 3.2 Other Instructions

### 3.2.1 Halt Instruction

| 000000 | 0000 | 00000000 |
| --- | --- | --- |
| 0 5 | 6 9 | 1 1 0 5 |

| Instruction: | Halt |
| --- | --- |
| Octal-Opcode: | 00 |
| Binary-Opcode: | 000000 |
| Function: | Stops the machine |