CISC SIMULATOR

Manual

V 2.0

Group 8

Md Shahjalal

Tianyou Bao

Xuzheng Lu

Contents

1 Introduction	1

1.1 Debugging Panel	1

1.1.1 Register Indicators Area	1

1.1.2 Memory Area	2

1.1.3 Controller Area	2

2.2 Classic Panel	3

2 Operation	3

2.1 Writing Values to Registers	3

2.2 Writing Values to Memory	3

2.2.1 Using Memory Address Register and Memory Buffer Register	3

2.2.2 Modifying the Memory Area	4

2.3 Executing Instructions	4

2.3.1 Executing Instructions Step-by-Step	4

2.3.1 Executing Instructions Automatically	5

3 Instructions Reference	6

3.1 Load/Store Instructions	6

3.1.1 LDR	6

3.1.2 STR	7

3.1.3 LDA	7

3.1.4 LDX	7

3.1.5 STX	7

3.2 Arithmetic and Logical Instructions	8

3.2.1 AMR	8

3.2.2 SMR	8

3.2.3 AIR	9

3.2.4 SIR	9

3.2.5 MLT	9

3.2.6 DVD	9

3.2.7 TRR	10

3.2.8 AND	10

3.2.9 ORR	10

3.2.10 NOT	10

3.3 Transfer Instructions	11

3.3.1 JZ	11

3.3.2 JNE	11

3.3.3 JCC	11

3.3.4 JMA	12

3.3.5 JSR	12

3.3.6 RFS	12

3.3.7 SOB	12

3.3.8 JGE	13

3.4 Shift/Rotate Instructions	13

3.4.1 SRC	13

3.4.2 RRC	14

3.5 I/O Instructions	14

3.5.1 IN	14

3.5.2 OUT	14

3.5.3 CHK	15

3.6 Other Instructions	15

3.6.1 HALT	15

# 1 Introduction

This simulator is a simulation of a Complex Instruction Set Computer (CISC). Two panels are designed for the simulator.

## 1.1 Debugging Panel

Debugging Panel displays all the information about the Registers, Indicators, and Memory in the computer and can be written manually.

The panel is divided into three parts：

### 1.1.1 Register Indicators Area

The Register Indicators display the values of all kinds of registers.

- Click the 'W' button to manually modify the value of a register.

- Hexadecimal values are shown on the right.

| Type | Size(bits) | Number | Description |
| --- | --- | --- | --- |
| R0...R3 | 16 | 4 | General-Purpose Register |
| IX1...IX3 | 16 | 3 | Index Register |
| MAR | 16 | 1 | Memory Address Register |
| MBR | 16 | 1 | Memory Buffer Register |
| PC | 12 | 1 | Program Counter |
| IR | 16 | 1 | Instruction Register |
| CC | 4 | 1 | Condition Code |
| MFR | 4 | 1 | Machine Fault Register |

### 1.1.2 Memory Area

The Memory Area shows the address, the value, the Hexadecimal value, and the Assemble Code of each line on memory.

- The memory address pointed by the Program Counter will be highlighted.

- Double click to manually modify the binary value of a memory row.

### 1.1.3 Controller Area

The Controller Area integrates all function buttons and the instruction input box.

| Button | Description |
| --- | --- |
| Reload | Initialize the values |
| Save | Save inputs |
| IPL | Pre-load a program |
| Auto Run | Run instructions automatically |
| Single Run | Run instructions step by step |
| Pause | Pause the machine |
| Stop | Stop the machine |
| Restart | Restart the machine |

## 2.2 Classic Panel

The appearance and operational logic of the Classic Panel emulate the PDP-8 computer. Users will use switches to input and lights for indication.

The Classic Panel has not been finished yet and will be released in the next version.

# 2 Operation

## 2.1 Writing Values to Registers

Following the steps below to write a value to a register.

Step 1: Input a value into the box

Step 2: Click the 'W' button at right to write the value to the register

Step 3: Done! The value will be written to the Register.

Error handling:

- Input too long: Remove the excess bits from the left

- Input too short: Add zeros from the left

- Input is not binary: Pop up an Error window

## 2.2 Writing Values to Memory

Two methods are acceptable to write a value to the Memory.

### 2.2.1 Using Memory Address Register and Memory Buffer Register

Step 1: Input a value into the MAR box

Step 2: Click the 'W' button of MAR

Step 3: Input a value into the MBR box

Step 4: Click the 'W' button of MBR

Step 5: Done! The value of MAR will be written to the Memory, and the MAR will automatically change to the next address.

### 2.2.2 Modifying the Memory Area

Step 1: Double click the memory row that needs to modify

Step 2: An input window as the following will pop up. Input the value that needs to write to the memory

Step 3: Click the 'OK' button, and then the value will be written to the Memory.

## 2.3 Executing Instructions

Instruction can be executed step-by-step or automatically.

### 2.3.1 Executing Instructions Step-by-Step

Step 1: Store an instruction to the Memory

Step 2: Write the address of the instruction to the Program Counter (PC)

Step 3: Click the 'Single Run' button, and then the instruction will be executed.

- The Program Counter will automatically point to the next address of Memory.

- The Instruction Register will store the last executed instruction.

### 2.3.1 Executing Instructions Automatically

Step 1: Store instructions to the Memory

Step 2: Write the address of the starting instruction to the Program Counter (PC)

Step 3: Click the 'Auto Run' button, and then the instructions will be executed automatically.

- The Program Counter will automatically point to the next address of Memory after an instruction being executed.

- All the indicators will be continuously updated while the program is running.

Step 4: Click the 'Pause' button or the 'Stop' button to stop the program.

# 3 Instructions Reference

## 3.1 Load/Store Instructions

The instructions to load/store values from/to Registers or Memory. The binary instruction code format of Load/Store Instructions is as follows:

| Opcode | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Opcode: | 6 bits | Specifies the instruction |
| --- | --- | --- |
| R: | 2 bits | Specifies the General-Purpose Register |
| IX: | 2 bits | Specifies the Index Register |
| I: | 1 bit | Specifies Indirect Addressing If I =1, indirect addressing; otherwise, no indirect addressing. |
| Address: | 5 bits | Specifies the location |

### 3.1.1 LDR

| 000001 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | LDR r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 01 |
| Binary-Opcode: | 000001 |
| Function: | Loads Register from Memory |

### 3.1.2 STR

| 000010 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | STR r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 02 |
| Binary-Opcode: | 000010 |
| Function: | Stores Register to Memory |

### 3.1.3 LDA

| 000011 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | LDA r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 03 |
| Binary-Opcode: | 000011 |
| Function: | Loads Register with Address |

### 3.1.4 LDX

| 101001 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | LDX x, address[, I] |
| --- | --- |
| Octal-Opcode: | 41 |
| Binary-Opcode: | 101001 |
| Function: | Loads Index Register from Memory |

### 3.1.5 STX

| 101010 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | STX x, address[, I] |
| --- | --- |
| Octal-Opcode: | 42 |
| Binary-Opcode: | 101010 |
| Function: | Stores Index Register to Memory |

## 3.2 Arithmetic and Logical Instructions

The instructions to perform most of the computational works in the machine. The binary instruction code format of basic Arithmetic and Logical Instructions is as follows:

| Opcode | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Opcode: | 6 bits | Specifies the instruction |
| --- | --- | --- |
| R: | 2 bits | Specifies the General-Purpose Register |
| IX: | 2 bits | Specifies the Index Register |
| I: | 1 bit | Specifies Indirect Addressing If I =1, indirect addressing; otherwise, no indirect addressing. |
| Address: | 5 bits | Specifies the location |

The binary instruction code format of register-to-register Arithmetic and Logical Instructions is as follows:

| Opcode | Rx | Ry |  |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 1 0 5 |

| Opcode: | 6 bits | Specifies the instruction |
| --- | --- | --- |
| Rx: | 2 bits | Specifies the General-Purpose Register x |
| Ry: | 2 bits | Specifies the General-Purpose Register y |

### 3.2.1 AMR

| 000100 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | AMR r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 04 |
| Binary-Opcode: | 000100 |
| Function: | Add Memory to Register |

### 3.2.2 SMR

| 000101 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | SMR r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 05 |
| Binary-Opcode: | 000101 |
| Function: | Subtract Memory from Register |

### 3.2.3 AIR

| 000110 | R |  |  | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | AIR r, immed |
| --- | --- |
| Octal-Opcode: | 06 |
| Binary-Opcode: | 000110 |
| Function: | Add Immediate to Register |

### 3.2.4 SIR

| 000111 | R |  |  | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | SIR r, immed |
| --- | --- |
| Octal-Opcode: | 07 |
| Binary-Opcode: | 000111 |
| Function: | Subtract Immediate from Register |

### 3.2.5 MLT

| 010100 | Rx | Ry |  |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 1 0 5 |

| Instruction: | MLT rx, ry |
| --- | --- |
| Octal-Opcode: | 20 |
| Binary-Opcode: | 010100 |
| Function: | Multiply Register by Register |

### 3.2.6 DVD

| 010101 | Rx | Ry |  |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 1 0 5 |

| Instruction: | DVD rx, ry |
| --- | --- |
| Octal-Opcode: | 21 |
| Binary-Opcode: | 010101 |
| Function: | Divide Register by Register |

### 3.2.7 TRR

| 010110 | Rx | Ry |  |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 1 0 5 |

| Instruction: | TRR rx, ry |
| --- | --- |
| Octal-Opcode: | 22 |
| Binary-Opcode: | 010110 |
| Function: | Test the Equality of Register and Register |

### 3.2.8 AND

| 010111 | Rx | Ry |  |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 1 0 5 |

| Instruction: | AND rx, ry |
| --- | --- |
| Octal-Opcode: | 23 |
| Binary-Opcode: | 010111 |
| Function: | Logical AND of Register and Register |

### 3.2.9 ORR

| 011000 | Rx | Ry |  |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 1 0 5 |

| Instruction: | ORR rx, ry |
| --- | --- |
| Octal-Opcode: | 24 |
| Binary-Opcode: | 011000 |
| Function: | Logical OR of Register and Register |

### 3.2.10 NOT

| 011001 | Rx |  |
| --- | --- | --- |
| 0 5 | 6 7 | 8 1 5 |

| Instruction: | NOT rx |
| --- | --- |
| Octal-Opcode: | 25 |
| Binary-Opcode: | 011001 |
| Function: | Logical NOT of Register to Register |

## 3.3 Transfer Instructions

The instructions to check the value of a register and then change the control of program execution.

The binary instruction code format of Transfer Instructions is as follows:

| Opcode | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Opcode: | 6 bits | Specifies the instruction |
| --- | --- | --- |
| R: | 2 bits | Specifies the General-Purpose Register |
| IX: | 2 bits | Specifies the Index Register |
| I: | 1 bit | Specifies Indirect Addressing If I =1, indirect addressing; otherwise, no indirect addressing. |
| Address: | 5 bits | Specifies the location |

### 3.3.1 JZ

| 001010 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | JZ r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 10 |
| Binary-Opcode: | 001010 |
| Function: | Jump if Zero |

### 3.3.2 JNE

| 001011 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | JNE r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 11 |
| Binary-Opcode: | 001011 |
| Function: | Jump if Not Equal |

### 3.3.3 JCC

| 001100 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | JCC cc, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 12 |
| Binary-Opcode: | 001100 |
| Function: | Jump if Condition Code |

### 3.3.4 JMA

| 001101 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | JMA x, address[, I] |
| --- | --- |
| Octal-Opcode: | 13 |
| Binary-Opcode: | 001101 |
| Function: | Unconditional Jump to Address |

### 3.3.5 JSR

| 001110 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | JSR x, address[, I] |
| --- | --- |
| Octal-Opcode: | 14 |
| Binary-Opcode: | 001110 |
| Function: | Jump and Save Return Address |

### 3.3.6 RFS

| 001111 |  |  |  | Immed |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | RFS immed |
| --- | --- |
| Octal-Opcode: | 15 |
| Binary-Opcode: | 001111 |
| Function: | Return from Subroutine with Return Code as Immediate Portion (optional) Stored in the Instruction’s Address Field |

### 3.3.7 SOB

| 010000 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | SOB r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 16 |
| Binary-Opcode: | 010000 |
| Function: | Subtract One and Branch |

### 3.3.8 JGE

| 010001 | R | IX | I | Address |
| --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 9 | 1 0 | 1 1 1 5 |

| Instruction: | JGE r, x, address[, I] |
| --- | --- |
| Octal-Opcode: | 17 |
| Binary-Opcode: | 010001 |
| Function: | Jump Greater than or Equal to |

## 3.4 Shift/Rotate Instructions

The instructions to manipulate a datum in a register. The binary instruction code format of Shift and Rotate Instructions is as follows:

| Opcode | R | A/L | L/R |  | Count |
| --- | --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 | 9 | 1 1 0 1 | 1 1 2 5 |

| Opcode: | 6 bits | Specifies the instruction |
| --- | --- | --- |
| R: | 2 bits | Specifies the General-Purpose Register |
| A/L: | 2 bits | Arithmetic Shift (A/L = 0); Logical Shift (A/L = 1) |
| L/R: | 2 bits | Logical Rotate (L/R = 1) |
| Count: | 4 bits | Specifies the Count for Operation |

### 3.4.1 SRC

| Opcode | R | A/L | L/R |  | Count |
| --- | --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 | 9 | 1 1 0 1 | 1 1 2 5 |

| Instruction: | SRC r, count, L/R, A/L |
| --- | --- |
| Octal-Opcode: | 31 |
| Binary-Opcode: | 011111 |
| Function: | Shift Register by Count |

### 3.4.2 RRC

| Opcode | R | A/L | L/R |  | Count |
| --- | --- | --- | --- | --- | --- |
| 0 5 | 6 7 | 8 | 9 | 1 1 0 1 | 1 1 2 5 |

| Instruction: | RRC r, count, L/R, A/L |
| --- | --- |
| Octal-Opcode: | 32 |
| Binary-Opcode: | 100000 |
| Function: | Rotate Register by Count |

## 3.5 I/O Instructions

The instructions to communicate with the peripherals attached to the computer system. The binary instruction code format of I/O Instructions is as follows:

| Opcode | R |  | DevID |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 81 0 | 1 1 1 5 |

| Opcode: | 6 bits | Specifies the instruction |
| --- | --- | --- |
| R: | 2 bits | Specifies the General-Purpose Register |
| DevID: | 5 bits | Device ID: 0 Console Keyboard 1 Console Printer 2 Card Reader 3-31 Console Registers, Switches, etc. |

### 3.5.1 IN

| 111101 | R |  | DevID |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 81 0 | 1 1 1 5 |

| Instruction: | IN r, devid |
| --- | --- |
| Octal-Opcode: | 61 |
| Binary-Opcode: | 111101 |
| Function: | Input Character to Register from Device |

### 3.5.2 OUT

| 111110 | R |  | DevID |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 81 0 | 1 1 1 5 |

| Instruction: | OUT r, devid |
| --- | --- |
| Octal-Opcode: | 62 |
| Binary-Opcode: | 111110 |
| Function: | Output Character to Device from Register |

### 3.5.3 CHK

| 111111 | R |  | DevID |
| --- | --- | --- | --- |
| 0 5 | 6 7 | 81 0 | 1 1 1 5 |

| Instruction: | CHK r, devid |
| --- | --- |
| Octal-Opcode: | 63 |
| Binary-Opcode: | 111111 |
| Function: | Check Device Status to Register |

## 3.6 Other Instructions

### 3.6.1 HALT

| 000000 |  |
| --- | --- |
| 0 5 | 6 1 5 |

| Instruction: | HALT |
| --- | --- |
| Octal-Opcode: | 00 |
| Binary-Opcode: | 000000 |
| Function: | Stop the machine |